CBAM Carbon Footprint Formulas

Iron & Steel and Aluminium Sectors — Reference Sheet

Compiled for academic/study use — July 2026

# 0. Scope and legal basis

The Carbon Border Adjustment Mechanism (CBAM) charges importers of certain carbon-intensive goods into the EU for the greenhouse gas emissions embedded in those goods. Iron & steel and aluminium are two of the six covered sectors (with cement, fertilisers, hydrogen and electricity). This sheet covers only the emissions-calculation formulas (the "carbon footprint" side) — not customs/reporting procedure.

Legal basis:

- Regulation (EU) 2023/956 — the CBAM framework regulation (definitions, scope, Annex I goods list, Annex II "direct-emissions-only" sectors, certificate formula in Art. 9/31).

- Implementing Regulation (EU) 2023/1773 — transitional-period reporting methodology (Annex III: calculation rules, system boundaries, equations).

- Implementing Regulation (EU) 2025/2547 — definitive-period calculation methodology (applies from 1 Jan 2026 imports).

- Implementing Regulation (EU) 2025/2620 / 2025/2621 — free-allocation adjustment method and default values for the definitive period.

- Underlying monitoring methodology is the same as the EU ETS Monitoring & Reporting Regulation, Commission Implementing Regulation (EU) 2018/2066 (fuel/mass-balance calculation methods, and Annex IV for the aluminium PFC methods).

Important: web access to the official EUR-Lex and European Commission PDF texts was blocked in this research session, so exact article/equation numbers below could not be re-verified word-for-word against the primary legal text in this session. The formulas themselves are cross-checked across multiple independent technical/consultancy sources describing the same official methodology (see Sources, section 5) and are standard, well-established industry methods (IPCC / EU ETS MRR). Before using this for a real compliance filing, open the regulation links in section 5 yourself and check the live article numbers, since implementing acts get amended.

# 1. General CBAM formulas (apply to every sector)

## 1.1 Total embedded emissions of an import

Embedded Emissionsₜₒₜₐₗ = Σg ( Qg × SEEg )

| Variable | Meaning |
| --- | --- |
| Qg | Quantity of good g imported in the reporting period (tonnes) |
| SEEg | Specific Embedded Emissions of good g (t CO₂e per tonne of product) |

## 1.2 Specific Embedded Emissions (SEE) of a good

SEEg = SEE(direct,g) + SEE(indirect,g)

For iron & steel and aluminium, both goods are listed in Annex II of Regulation (EU) 2023/956, meaning only direct emissions are priced under CBAM:

SEEg  =  SEE(direct,g)      (indirect/electricity term = 0 for steel & aluminium)

Cement and fertilisers are the sectors where indirect (electricity) emissions are also priced. For steel and aluminium, electricity consumption is tracked for reporting/statistics but is currently not charged.

## 1.3 Direct emissions of a source stream — calculation-based (Standard) method

Ei = ADi × EFi × OFi   (fuels / combustion)

Ei = ADi × EFi × CFi    (process materials, non-combustion)

| Variable | Meaning |
| --- | --- |
| ADi | Activity data: quantity of fuel or material i consumed (t, Nm³, or GJ) |
| EFi | Emission factor of fuel/material i (t CO₂ per t, per Nm³, or per GJ) |
| OFi | Oxidation factor (fraction of carbon actually oxidised in combustion, ≈0.98–1) |
| CFi | Conversion factor (fraction of carbon converted to CO₂ in a process reaction) |

## 1.4 Direct emissions — Mass-balance method (used for multi-input processes, common in steel)

E(CO2) = [ Σ(Minput × wC,input) − Σ(Moutput × wC,output) ] × 44/12

| Variable | Meaning |
| --- | --- |
| Minput / Moutput | Mass of each material flowing into / out of the process (t) |
| wC | Carbon content of that material (fraction, t C / t material) |
| 44/12 | Molar mass ratio converting mass of carbon to mass of CO₂ |

## 1.5 Specific embedded emissions of an installation's own good

SEE(direct,g) = AEEg / ALg

| Variable | Meaning |
| --- | --- |
| AEEg | Attributed direct emissions of good g in the reporting period (t CO₂e) — the share of the installation's total measured emissions assigned to g |
| ALg | Activity level: quantity of good g actually produced in the reporting period (t) |

## 1.6 Complex / downstream goods (embedded emissions of precursors carry through)

SEE(good) = SEE(own process) + Σp ( SEE(precursor p) × ratio p )

| Variable | Meaning |
| --- | --- |
| SEE(own process) | Direct emissions from the good's own final production step only (e.g. rolling, drawing, extrusion, casting) |
| SEE(precursor p) | Specific embedded emissions already carried by precursor material p (e.g. crude steel, primary aluminium, alloy) |
| ratio p | Tonnes of precursor p consumed per tonne of final good produced (accounts for process yield / scrap losses) |

This is the mechanism by which emissions accumulate down the value chain: a finished steel bar 'inherits' the footprint of the crude steel, which inherits the footprint of the pig iron/DRI, which inherits the footprint of sintered ore, etc.

# 2. Iron & Steel sector

Steel production is split into aggregated goods categories (system boundaries defined in Annex I): sintered/pelletised ore, pig iron, ferro-manganese, ferro-chrome, ferro-nickel, DRI (direct reduced iron), crude steel, and downstream finished products (bars, rebar, wire rod, plates, coils, tubes, etc.). Only direct emissions count (section 1.2).

## 2.1 Sinter/pellet plant

E = Σ(AD(coke breeze, fuel gas) × EF × OF) + E(carbonate calcination, mass balance)

Process emissions come mainly from combustion of coke breeze/fuel gas plus calcination of limestone/dolomite flux, calculated with the mass-balance formula (1.4).

## 2.2 Blast furnace — pig iron

E = Σ(AD(coke, PCI coal, other reductants) × EF × OF) + E(ore reduction, mass balance)

Applies formula 1.3 to purchased/combusted fuels and formula 1.4 (carbon in coke/coal/ore vs. carbon out in hot metal + blast-furnace gas + slag) for the reduction chemistry.

## 2.3 Basic Oxygen Furnace (BOF) — crude steel from hot metal + scrap

E(CO2) = [ (Mhotmetal×wC,hm + Mscrap×wC,scrap + Mflux×wC,flux) − Msteel×wC,steel ] × 44/12

Pure mass-balance process: carbon-rich hot metal (~4.3% C) is decarburised to steel (~0.05–1% C); the carbon difference leaves as BOF gas/CO₂.

## 2.4 Electric Arc Furnace (EAF) — crude steel from scrap/DRI

E(CO2) = Σ(Mcarbon inputs × wC) − Msteel×wC,steel) × 44/12  +  Σ(ADnat.gas/electrode × EF × OF)

Main direct-emission sources: graphite electrode consumption, injected carbon/natural gas for foaming slag, and carbon in scrap/DRI vs. carbon in steel output. Electricity used to melt the charge is an indirect emission and — per section 1.2 — is not priced for steel.

## 2.5 Direct Reduced Iron (DRI)

E = AD(natural gas or syngas) × EF × OF   (combustion + reduction reaction, gas-based route)

## 2.6 Downstream finished steel products

SEE(finished product) = SEE(crude steel) × ratio(crude steel/product) + SEE(own process, e.g. rolling)

ratio(crude steel/product) is the yield factor: tonnes of crude steel needed per tonne of finished product, which is >1 because of scale/trim losses.

# 3. Aluminium sector

Aluminium is also Annex II (direct emissions only), but unlike steel it additionally involves perfluorocarbon (PFC) gases (CF₄, C₂F₆) from the electrolysis process, which must be converted to CO₂-equivalent using GWP factors.

## 3.1 Primary aluminium — CO₂ from anode consumption

### (a) Simple/default method

E(CO2) = EFtechnology × Production(Al)

EFtechnology is a default factor (t CO₂ / t Al) that depends on the cell technology: prebake, Søderberg vertical-stud (VSS) or horizontal-stud (HSS).

### (b) Mass-balance / net carbon consumption method (more accurate, plant-specific)

E(CO2) = Production(Al) × NCC × 44/12

| Variable | Meaning |
| --- | --- |
| Production(Al) | Tonnes of primary aluminium produced in the period |
| NCC | Net Carbon Consumption: net mass of anode carbon actually oxidised to CO₂ per tonne of Al produced (t C / t Al), i.e. gross anode consumption minus recovered butts/ash and non-oxidised carbon |

## 3.2 Primary aluminium — PFC emissions (CF₄ and C₂F₆ from anode effects)

### Tier 2 — Slope method

E(CF4) [kg/t Al] = S(CF4) × AEM

E(C2F6) [kg/t Al] = E(CF4) × F(C2F6/CF4)

| Variable | Meaning |
| --- | --- |
| S(CF4) | Slope coefficient, measured per smelter/technology — (kg CF₄ / t Al) per (anode-effect-minute / cell-day) |
| AEM | Anode Effect Minutes per cell-day = frequency of anode effects × average duration (minutes/cell-day) |
| F(C2F6/CF4) | Technology-specific mass ratio of C₂F₆ to CF₄ generated (typically ~0.1 for center-worked prebake cells) |

### Tier 3 — Overvoltage method

E(CF4) [kg/t Al] = K × AEO / CE

| Variable | Meaning |
| --- | --- |
| K | Overvoltage coefficient, technology-specific (kg CF₄ / t Al per mV of anode-effect overvoltage) |
| AEO | Anode Effect Overvoltage — average excess cell voltage during anode effects × frequency (mV/cell-day) |
| CE | Current efficiency of the cell (fraction, typically ~0.90–0.96) |

### Converting PFC mass to CO₂-equivalent

CO2e(PFC) = [ E(CF4)×Production(Al) ] × GWP(CF4)  +  [ E(C2F6)×Production(Al) ] × GWP(C2F6)

| Variable | Meaning |
| --- | --- |
| GWP(CF4) | 100-yr Global Warming Potential of CF₄ = 6,630 (IPCC AR5) |
| GWP(C2F6) | 100-yr Global Warming Potential of C₂F₆ = 11,100 (IPCC AR5) |

Total direct SEE of primary aluminium = [E(CO2) from 3.1 + CO2e(PFC) from 3.2] ÷ Production(Al).

## 3.3 Secondary (recycled) aluminium

E(CO2) = Σ(AD(remelt furnace fuel) × EF × OF)

No electrolysis and no anode ⇒ no PFCs and no anode-CO₂ term. Emissions come only from combustion in the remelting/holding furnace, which is why recycled aluminium's footprint is typically far lower per tonne than primary aluminium (order of magnitude difference — treat exact benchmark numbers as indicative defaults published by the Commission, not fixed constants).

## 3.4 Downstream aluminium products (alloys, extrusions, rolled/cast products)

SEE(product) = SEE(primary or secondary Al input) × ratio(input/product) + SEE(own process, e.g. casting/extrusion/rolling)

# 4. From footprint to CBAM certificates (context)

Once SEE is known, the number of CBAM certificates an importer must surrender follows Regulation (EU) 2023/956, Art. 9 and 31:

N = [ SEEg − (Benchmarkg × CBAMFactoryear) ] × Qg

Nfinal = N × (1 − Carbon Price Adjustment)

| Variable | Meaning |
| --- | --- |
| Benchmarkg | EU ETS free-allocation benchmark for good g (t CO₂e/t) — reflects what an efficient EU producer would emit |
| CBAMFactoryear | Phase-out factor of free allocation: 2.5% (2026) → 5% (2027) → 10% (2028) → … → 100% (2034) |
| Carbon Price Adjustment | Deduction for a verified carbon price already effectively paid in the country of production |

This section is included for context only — the user's request was specifically about the emissions/footprint formulas in sections 1–3, which feed directly into this certificate calculation.

# 5. Sources

Primary legal texts (open these directly to check current article/equation numbering — they could not be fetched live in this session):

- Regulation (EU) 2023/956 (CBAM framework): https://eur-lex.europa.eu/eli/reg/2023/956/oj

- Implementing Regulation (EU) 2023/1773 (transitional-period methodology, Annex III): https://eur-lex.europa.eu/eli/reg_impl/2023/1773/oj

- Implementing Regulation (EU) 2025/2547 (definitive-period calculation methodology): search EUR-Lex CELEX 32025R2547

- Implementing Regulation (EU) 2025/2620 (free-allocation adjustment) / 2025/2621 (default values): search EUR-Lex CELEX 32025R2620 / 32025R2621

- EU ETS Monitoring & Reporting Regulation (EU) 2018/2066, Annex IV (fuel/mass-balance and aluminium PFC methods): https://www.legislation.gov.uk/eur/2018/2066/annex/IV

- IPCC Good Practice Guidance — PFC Emissions from Primary Aluminium Production: https://www.ipcc-nggip.iges.or.jp/public/gp/bgp/3_3_PFC_Primary_Aluminium_Production.pdf

Secondary sources used to cross-check and phrase the formulas (consultancy/industry explainers of the same official methodology):

- European Commission, Taxation and Customs Union — CBAM Guidance for installation operators: taxation-customs.ec.europa.eu

- CBAM Guide — Embedded Emissions Step-by-Step Calculation Guide 2026: cbamguide.com/compliance/embedded-emissions

- GTG Consulting — Mass Balance Method in CBAM (steel): gtgcons.com

- CarbonChain Help Center — Preparing CBAM data for Crude Steel, DRI, Pig Iron, Sinter: learn.carbonchain.io

- Decerna — CBAM Aluminium Calculator (primary vs secondary): decerna.co.uk

- International Aluminium Institute — Aluminium Sector Greenhouse Gas Protocol: ghgprotocol.org

- carboneer — CBAM definitive period: key regulatory updates and scope extension: carboneer.earth